DC-S6261X

Architectural and Engineering Specifications

Version 1.1

(Mar, 26, 2024)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Section 28 23 29 – Video Surveillance Remote Devices and Sensors

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13493, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DC-S6261X is an indoor Network WDR Mini PTZ Camera (IP Camera) designed and manufactured by IDIS. This camera provides 2MP (1920 x 1080) resolution at 30ips (images per second) with H.265, H.264, and MJPEG compression. This camera is equipped with 12x Auto Focus/Zoom lens, PoE (IEEE 802.3af Class 3), Audio I/O, Alarm I/O, micro SD/SDHC/SDXC card backup.

General Specification

- The IP camera shall be equipped with 2 Megapixel 1/2.8” CMOS Sensor.

- The IP camera shall be equipped with 5.3mm – 64mm auto-focus zoom lens, F1.6 – F2.8.

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have wide dynamic range compensation (True WDR) for improved video quality in high-contrast situations (120dB).

- The IP camera shall support wide pan (360o) and tilt (0o ~ 180o) range as well as zoom up to 12x.

- The IP camera shall utilize 2DNR or 3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The IP camera shall be equipped with 10/100 Base-T, auto-sensing, half/full duplex, RJ-45 Ethernet connection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af Class 3 to supply power to the camera over the network, 12VDC input.

- The IP camera shall have on board microSD/SDHC/SDXC card backup storage slot as a safeguard against data loss during network interruptions.

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 1920 x 1080 at rates up to 30ips (images per second).

- The IP camera shall provide direct network connection using H.265, H.264 and MJPEG compression.

- The IP camera shall support quadruple streams.

- The IP camera shall conform to the ONVIF Profile S/T.

- The IP camera shall have SSL Encryption, Multi-user Authority, IEEE 802.1x IP Filtering, HTTPS and configurable user authority levels for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Video Motion Detection, Active Tampering Alarm.

- The IP camera shall support De-fog function.

Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and M-JPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/2.8” CMOS

- Maximum Resolution: 1920 x 1080

- Scanning Mode: Progressive Scan

- Lens Type: AF(Auto Focus) Zoom Lens (f=5.3mm – 64mm, F1.6 – F2.8)

- Iris Control: DC Auto Iris (with hall sensor)

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - Wide: 58.2º(H), 34º(V), 65.3º(D)

  - Tele: 4.83º(H), 2.8º(V), 5.7º(D)

  - Pan Range, Speed: 360°, 0.02~150°/sec (preset 150°/sec)

  - Tilt Range, Speed: 180° (0° ~ 180°), 0.02~150°/sec (preset 150°/sec)

  - Zoom Range: x12

- Minimum Illumination:

  - Color: 0.1 lux @ F1.6

  - B/W: 0.01 lux @ F1.6

- S/N Ratio: more than 45dB

- Maximum Frame Rate: 30ips @1920 x 1080

- Video Resolution: DirectIP Protocol Mode: 1920 x 1080, 1280 x 720, 640 x 360, 352x240

- Video Compression : H.265, H.264, MJPEG

- Video Compression Level: 4 levels: Basic, Standard, High, Very High

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120dB (True WDR)

- True Day & Night: Yes (ICR)

- Image Stabilizer: Yes (EIS)

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm

- Analog Video Output: None

- Advanced PTZ options:

  - The number of preset: 256ea

- De-fog : Yes

Audio Specification

- Audio Compression Algorithm: ADPCM 16K, G.726, G.711 u-Law, G.711 a-Law

- Audio Input / Output: Line-in 1ea / Line-out 1ea

- Two-way Audio Communication: Yes

- Pre-recorded Voice Alert: Yes

Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols: DirectIP 2.0 Protocol, IPv4, IPv6, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP,

RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP, SNMPv2

- Streaming Mode: Quadruple Streaming

Security Specification

- SSL Encryption, Multi-user Authority, IEEE 802.1x IP Filtering, HTTPS

- Maximum User Access: 10 (Live), 1 (Recording), 1 (Search), 2 (Admin)

Alarm and Event Specification

- Alarm Input / Output: 4 / 2

  - Alarm Input: 4 TTL, NC/NO Programmable, 4.3V(NC) or 0.3V(NO) threshold, 5V DC

  - Mechanical or electrical switches can be wired to the Alarm-In and GND connectors. The maximum voltage should not exceed 5V.

  - Alarm Output: 2 TTL open collector, 30mA @ 5 VDC

- Trigger Events: Motion Detection, Alarm in, Audio detection, Tampering, Trip Zone

- Event Notification: Remote S/W, Email (with Image)

  - Encryption Type: SSL

Environmental Specification

- Operating Temperature: -10°C ~ +50°C (+14°F ~ +122°F)

  - starting up at above 0°C (32°F)

- Operating Humidity: 0% to 90%

- Vandal-proof Enclosure: No

Electrical Specification

- Power Source: 12VDC, PoE(IEEE 802.3af Class 3)

- Power Consumption:

  - 12VDC : 9.2W

  - PoE(IEEE 802.3af Class 3) : 10W

- Regulatory Approvals: FCC, CE (50130-4), KC, UL

Mechanical Specification

- Dimensions (Ø x H): Ø155mm x 158.1mm (Ø6.1" x 6.22")

- Unit Weight: 1.0 kg

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | TS Team | Jul, 15, 2021 | Initial Release |
| 1,1 | Roy Lee | Mar. 26. 2024 | Added Onvif T, De-fog function |