DC-T6224HRXL

Architectural and Engineering Specifications

Version 1.0

(Jul. 05, 2023)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Section 28 23 29 – Video Surveillance Remote Devices and Sensors

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 463-400, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DC-T6224HRXL is a Network Camera (IP Camera) designed and manufactured by IDIS. This camera provides Full HD (1920x1080) resolution at 30ips (images per second) with H.265/H.264/M-JPEG compression. This camera is equipped with Motorized Vari-focal lens, True Day/Night, IR LED, Audio I/O, Alarm I/O, IP66 rated and heater provide continuous operation in subzero temperature.

General Specification

- The IP camera shall be equipped with 2 Megapixel 1/2” CMOS Sensor.

- The IP camera shall be equipped with 12mm - 50mm Motorized Vari-focal lens, F1.5 – 1.9

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance. The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have Wide Dynamic Range compensation (Digital WDR) for improved video quality in high-contrast situations (120dB).

- The IP camera shall be equipped with Infrared LED with range up to 20m(License Plate capture up to 80km/h) and 100m(Surveillance).

- The IP camera shall utilize configurable 2DNR/3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The IP camera shall be IP rating 66 and heater.

- IP camera shall be equipped with 10/100 Base-T, auto-sensing, half/full duplex, RJ45 Ethernet connection.

- The IP camera shall support industry standard to supply power to the camera 12VDC input.

- The IP camera shall have built-in heater for continued use in subzero temperature conditions

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 1920x1080 at rates up to 30ips

(Images per second).

- The IP camera shall provide direct network connection using H.265, H.264 and M-JPEG** compression (** IDIS Protocol only).

- The IP camera shall support Quadruple Streams in DirectIP protocol mode.

- The IP camera shall support Quadruple Streams in IDIS protocol mode.

- The IP camera shall conform to the ONVIF** Profile S.

- The IP camera shall be equipped with embedded web server (IDIS Web**) which works independently using a Web Browser with ActivX plug-in (** IDIS Protocol only).

- The IP camera shall have IP filtering, HTTPS, SSL, IEEE 802.1X, and configurable user authority levels for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Video Motion Detection, Active Tampering Alarm, Trip Zone, Line Cross, Loitering and Object Detection.

Protocol Specification: DirectIP and IDIS Protocol

- The IP camera shall have 2 protocol modes, DirectIP and IDIS Protocol, and DirectIP is set as main protocol by default.

- The protocol modes shall be selectable between DirectIP and IDIS protocol mode to meet specific needs with IDIS Discovery tool.

  - DirectIP Protocol

    - DirectIP protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

    - DirectIP protocol shall provide Quadruple streams.

    - The bitrate shall be automatically adjusted by recording profile of DirectIP NVR.

    - DirectIP protocol shall support H.265 and H.264 only as primary compression.

  - IDIS Protocol

    - IDIS protocol shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

    - IDIS protocol shall provide Quadruple streams.

    - IDIS protocol shall support H.265, H.264 and M-JPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/2" CMOS

- Maximum Resolution: 1920 x 1080

- Scanning Mode: Progressive Scan

- Lens Type: Motorized Vari-focal (f= 12 - 50mm, F1.5 – 1.9)

- IRIS Control: P-Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - Wide : 3º(H), 18º(V), 38º(D)

  - Tele : 9º(H), 5º(V), 10º(D)

- Minimum Illumination:

  - COLOR : 0.02 lux @ F1.5

  - B/W : 0 lux (IR LED ON)

- S/N Ratio: More than 45dB

- Maximum Frame Rate:

  - 30ips @ 1920 x 1080

  - 25ips @ 1920 x 1080

- Video Resolution:

  - 1920x1080, 1280x720, 704x480, 640x360, 352x240

- Video Compression : H.265, H.264, M-JPEG

- Video Compression Level: Basic, Standard, High, Very High

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120dB

- True Day & Night: Yes (ICR)

- IR Distance (The number of LEDs):

License Plate capture up to 80km/h : 20m

Surveillance : 100m

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm, Trip Zone

Audio Specification

- Audio Compression Algorithm: ADPCM 16k, G.726, G.711 u-Law, G.711 a-Law

- Audio Input / Output: Line-in 1ea / Line-out 1ea

- Two-way Audio Communication: Yes

- Pre-recorded Voice Alert: Yes

Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols:

  - DirectIP Protocol Mode

  - IDIS Protocol Mode: RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP, RTP/UDP, RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP

- Streaming Mode: Unicast, Multicast

Security Specification

- DirectIP Protocol Mode: SSL Encryption

- IDIS Protocol Mode: Multi-User Authority, IEEE 802.1x, IP Filtering, HTTPS, SSL Encryption

- Maximum User Access:

  - DirectIP protocol mode: Direct camera access is unavailable.

  - IDIS protocol mode: 10 (Live), 1 (Recording), 1 (Search), 2 (Admin)

Alarm and Event Specification

- Alarm Input / Output : 2 / 1

  - Alarm Input: 2 TTL, NC/NO programmable, 4.3V (NC) or 0.3V (NO) threshold, 5 VDC

  - Mechanical or electrical switches can be wired to the Alarm-In and GND connectors. The maximum voltage should not exceed 5V.

  - Alarm Output: 1 TTL open collector, 30mA @ 5 VDC

- Trigger Events: Motion Detection, Alarm in, Audio Detection, Tampering, Trip Zone, Line Cross, Loitering, Object Detection

- Event Notification: Remote Software, Email (with Image)

  - Encryption type: SSL, TLS

Environmental Specification

- Operating Temperature: -40°C ~ +60°C (-40°F ~ +140°F)

- Operating Humidity: 0% ~ 90%

- Outdoor Ready: IP66, Heater

Electrical Specification

- Power Source: 100V ~ 240VAC

- Power Consumption: 100-240V~, 50/60Hz, 1.0-0.5A, 50W

- Regulatory Approvals: FCC, CE, KC

Mechanical Specification

- Dimensions (Ø x H): Ø166.5mm x 129.5mm x 540mm(Ø6.56" x 5.1" x 21.26”)

- Unit Weight: 4.65 kg (10.25 lb)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | TS Team | Oct. 22. 2019 | Initial Release |
| 1.1 | TS Team | Apr. 28, 2021 | Spec Update |